Writing ST-X/PixCel Control Programs in Borland Delphi®

SBIG has long provided third-party development support for their line of astronomical CCD cameras. This support has presumed that the developer was using one of three common development environments: VisualBasic, Microsoft Visual C++ , or Borland’s C/C++ compiler version 4.5x. However, with only a little effort, one can also develop control applications using the 32-bit Borland Delphi RAD development environments.

While initial efforts were successful using Delphi 1, a 16-bit compiler, programming the handling of large image files is much easier under 32-bit. Only the latter will be considered here.

# The ObjectPascal “Header”

The first step to writing a program for Delphi is to convert the header file (PARDRV.H). This means converting the function and procedure calls as well as data types to be compatible with the ObjectPascal language. The resulting file is called a “unit”. In the simplest of terms, a unit is a combined H and C/CPP file. The declarations (what would be in the H file) are made in what’s called the interface section. The actual code (what would be in the C/CPP file) is in the implementation section of the unit. Once written, the file is added to the “uses” list in any other unit that requires access to the declarations and/or implementation of the functions and procedures. This is akin to placing the given H file in the “includes” list of a C/CPP file.

An important consideration is making sure that the C data types in the original file are properly converted, not only in terms of being a signed or unsigned integer, but in storage size as well. For example, a SHORT in C/C++ is the same as a SmallInt in ObjectPascal, i.e., both are unsigned integers with a range of –32767 to +32768 and occupying two bytes in memory.

Another consideration is parameters passed by reference. There are several ways to accomplish this in the OP language. In some cases, a parameter was declared as a var, meaning if its value was modified within the function or procedure, the variable would take on the new value after the function or procedure returned. Another is passing the parameter as a pointer to the appropriate data type. This is the method used by the author since the ParDrvCommand is expecting an untyped pointer for one or two of its three parameters.

C/C++ does not have procedures, only functions. In ObjectPascal, a procedure is the same as a C/C++ void function, i.e., a function that does not return a value.

# Using the Microsoft Version Files

For various logical reasons, SBIG has dropped support for the Borland C/C++ compilers. This means that the Microsoft version of the DLL (SBIG32M.DLL) must be used. In the case of the parallel driver, there is one and only one exported function call in the DLL, ParDrvCommand, which takes three parameters.

In the ObjectPascal unit, the function is declared using the cdecl calling convention and referencing the function by name, i.e.,

function ParDrvCommand(Command : ShortInt;

Params  : Pointer;

Results : Pointer) : ShortInt; cdecl; far;

external 'SBIG32M.DLL' index 1;

Technically, the far keyword is not required in 32-bit, but the code is a carry over from the 16-bit version. Including the far keyword does not affect the code as the compiler ignores it. The SBIG32M.LIB export library file supplied with the SBIG files is not used when working in Delphi. It’s sole function being to export the function in the DLL. That is handled in the ObjectPascal unit as described above.

# Working with the ParDrvCommand Function in Delphi

To get you started, a few examples of how to work with the ParDrvCommand function call within a Delphi program are shown below.

# Linking to the Camera

The most obvious first step is to be able to link to the camera. By including the export in the Pascal header unit, it’s possible to force the DLL to load at startup, meaning calls to the Windows API LoadLibrary and FreeLibrary are not required and the DLL occupies the same space for the duration the application is run. Before trying to link, you need to initialize the driver. This can be done with the following line of code in the main form’s FormCreate method.

Idx := ParDrvCommand(17, nil, nil);

if (Idx = 0) then

stbCamera.Panels[0].Text := 'Parallel driver opened'

else

stbCamera.Panels[0].Text := 'Failed to open parallel driver: '

+ IntToStr(Idx);

The first line attempts to initialize the library passing command 17 to the function. The return value of the function call determines which text is set into a status bar panel.

The next step is to call the ParDrvCommand function with the appropriate parameters.

function TfrmMain.LinkCamera(Link : Boolean) : boolean;

var

Return : ShortInt;

ELP    : TEstablishLinkParams;

ELR    : TEstablishLinkResults;

begin

ELP.Port := 0;

ELP.BaseAddress := PPort[cbxCameraPort.ItemIndex];

FillChar(ELR, SizeOf(TEstablishLinkResults), #0);

if (ELP.BaseAddress = 0) then begin

stbCamera.Panels[0].Text := 'Error reading LPT port address';

Result := False;

Exit;

end;

Return := ParDrvCommand( 9, @ELP, @ELR);

if (Return <> 0) then begin

stbCamera.Panels[0].Text := 'Error linking to camera';

Result := False;

end else begin

{do camera setup, e.g., to use last settings}

Result := True;

end;

end;

The line

ELP.BaseAddress := PPort[cbxCameraPort.ItemIndex];

Sets the base address for the parallel port, usually 3F8 hex. for LPT1. During program startup, you can spawn the example program supplied by SBIG (LPTADDRS) to find the parallel port addresses and store them in an array.

## Taking an Exposure

The example below is very basic. It directs the camera to take a 1 second exposure using the main guiding chip at 1x1 binning.

function TfrmST78.TakeST78LightImage : Boolean;

var

SEP    : TStartExposureParams;

EEP    : TEndExposureParams;

RLP    : TReadoutLineParams;

QCP    : TQueryCommandStatusParams;

QCR    : TQueryCommandStatusResults;

ERP    : TEndReadoutParams;

Return : ShortInt;

I,

J,

IHeight: Longint;

Buf    : PByteArray;

begin

{set the StartExposure values}

SEP.CCD := 0;

SEP.ExposureTime  := 100;

SEP.OpenShutter := 1;

{Make sure the QueryResults record is all 0’s}

FillChar(QCR, SizeOf(QCR), #0);

{start the exposure}

Return := ParDrvCommand( 1, @SEP, nil);

if (Return = 0) then begin

{the command to start the exposure was accepted}

{query the camera to determine when the end exposure flag is set}

QCP.Command := 1;

repeat

ParDrvCommand(12, @QCP, @QCR);

{Application.ProcessMessages keeps the loop from tying up the CPU}

{while the program “waits” for the exposure to end}

Application.ProcessMessages;

until (FlagIsSet(QCR.Status, $0002) and FlagIsSet(QCR.Status, $0001));

pnlStatus.Caption := 'Light Exposure: Completed';

{send the EndExposure command to the camera}

EEP.CCD := 0;

ParDrvCommand( 2, @EEP, nil);

{setup the ReadoutLine record}

RLP.CCD := 0;

RLP.PixelStart := 0;

RLP.ReadoutMode := 0;

{CamInfo can be global or passed as a parameter. It is a record}

{that describes the desired setup for the camera, e.g., binning, etc.}

RLP.PixelLength := CamInfo.ImageWidth;

IHeight := CamInfo.ImageHeight;

{MLiteImg is an object of a class defined elsewhere in the program}

{This class allows reading, displaying, and saving of several image}

{formats, including FITS, SBIG, BMP, TIF, and JPEG}

{The CreateEmptyFITS method creates a 16-bit FITS compatible header}

{and buffer.}

{The PixBuf field of the class allows reading and writing to the}

{image buffer. The temporary variable, Buf, is used to make passing}

{a pointer to the ParDrvCommand function easier. No memory is}

{allocated to Buf, it simply holds the address of the image buffer}

{in the MLiteImg object}

MLiteImg.CreateEmptyFITS(CamInfo.ImageHeight, CamInfo.ImageWidth);

Buf := MLiteImg.PixBuf;

for I := 0 to pred(IHeight) do begin

J := I * CamInfo.ImageWidth * 2;

ParDrvCommand( 3, @RLP, @Buf[J]);

end;

ERP.CCD := 0;

ParDrvCommand(25, @ERP, nil);

Result := True;

end;

end;

The FlagIsSet is a function that tests if a given bit is set within set of one or more bytes.

### Summary

The above examples along with the provided USBCONST.PAS “header” file and documentation provided by SBIG should allow you to create a quick, simple program to control the SBIG ST-7/8/9 and PixCel cameras using the parallel port driver. Extending a program to use more of the available commands via the ParDrvCommand function is not overly difficult. The challenge is writing a bullet-proof user interface that anticipates difficulties and improper user input.

Brian D. Warner

716 Palmer Divide Observatory

17995 Bakers Farm Rd.

Colorado Springs, CO  80908

brianw_mpo@compuserve.com